| # Fixing Plastic Sheeting to Wood | # Fixing to Rope |
| --- | --- |
| ; . Nail will tear sheet; bending nails, using washers or bottle caps is better; . ; ; Double fold sheet over wood before fixing. Too close to edge; Fold sheet over wood before fixing.  Fixtures too far;  fixtures 6”-12” apart. |  no reinforcement.  rope through eyelet.  rope through reinforcement band.  place stone inside.  make knot with rope inside. |

# Fixing Plastic Sheeting to Bamboo

 nailed straight through plastic.  wired (better) or nailed through section of bamboo;  plastic sheeting folded over then wired (better) or nailed through section of bamboo.  wire looped around batten passed through hole in the sheet and wrapped around the pole;  plastic sheet folded back on itself, then wire looped around batten passed through hole in the sheet and wrapped around the pole.